ALROSE

Begin at the intersection of Arizona State Route No. 288 and USFS road No. 54A;

Go South on USFS road No. 54A;

0.70 miles – cattleguard;

0.704 miles – turn left on Hog Canyon Road to North and Northeast;

0.92 miles – top of first hill from road intersection with USFS Road No. 54A –  point is on South side of old barbed wire fenceline running East and West – point is a 9/16 stainless steel rod in sleeve, driven to bedrock at 2.74 meters, depth. Sleeve is in concrete with an NGS cast aluminum lid at natural ground level.