Kinney Aerial – John Cahoon	, Rick Bunger, Tom Homan		6/17/04

32 SQ miles aerial. 2ft topo project.

GPS Control of flight

4 units min 6+ better running at 1sec collection rates

12-15 panels down for checks, etc.

RFP might include having Surveyor who has worked w/ Airborne GPS software before vs. us processing it ourselves. We have the software already so no additional purchases would need to be made.

Profiling – Done every 3rd stereo model

Looking at the control we have planned in the area, John restates panel counts down to 6 on the corners. Would we need panels on the new monuments?

Deliverables

- Planimetric

  - 200 scale

  - House

  - Roads

  - Tree Density

  - Major Features

  - ¼ section files

  - Autocad 2000 format

  - Overlapped files vs. butt joint

  - Seamless mosaic

    - SID compression @ 15:1

- DTM

- TIN

- Orthos

  - 0.5ft pixels

  - ¼ section files

  - Overlapped files vs. butt joint

  - Seamless mosaic

    - SID compression @ 15:1

- Metadata

Blind checks (panel section corners?)

Various panels placed on ground not used for control but validation

Approximate location provided to mapping co prior to flight

Should we panel these as ‘Y’ panels vs. ‘X’ for main panels?

Price differences between Color/BW?

- Color would allow better forest health determination, wildfire stuff, etc so higher reuse of data